Project 3: Blood Clotting: Platelet aggregation model & Particle Methods

Nicolas Garcia Callejas

03/31/2026

In this project we were asked to create a blood clotting simulation utilizing particle methods. The particles themselves were subjects of adhesive, drag, and contact forces as well as other unique parameters presented between the RBCs and PLTs. To further explain more about this implementation without going overly in detail; we can start by explaining some parameters such as the vessel wall. The vessel wall is represented in 2D just like every particle in this project, initially I had the vessel walls represented as stacks of RBCs, however due to some issues with this approached I instead took inspiration from working models of other’s presentation and use a finite line to represent an upper and lower wall. RBCs and PLTs were treated as small circles that moved according to blood flow, their reactions between contact and the PLTs activation and adhesion when near a threshold of a damaged area. When the simulation starts all particles are placed in random positions and scatter due to contact forces, avoiding overlap, all near the inlet of the vessel, once particle leave length L, they are recycled into the vessel unless PLTs go into an activated state which has them adhered into the damaged area. Blood flow is modeled using Poiseuille’s velocity profile which can be seen at time step 2000, as already known by this profile it means that the center moves faster while the upper and lower areas move slower, which can also make some unlucky PLTs in the upper wall move very slowly and not activate, as seen in the animation. When particles collide such as PLTs colliding with RBCs due to the adhesion force of the damage area being strong, we can see contact forces in effect making the RBCs bounce around. As shown in the animations, increasing the lesion side increased the number of activated PLTs, and vise-versa, as well as adding more PLTs would increase the number of activated PLTs. The biggest difference visually too was increasing the number of RBCs being simulated, which got in the way of some PLTs  preventing them from adhering to the damaged site. It is important to also mention that the timestep was calculated inside the loop with an initial timestep of 0.1 was tested, this correction was recommended by the professor to properly calculate the forces during each iteration and for the particles to not overcome their radius at each timestep. The main limitation of my current model is the lack of contact forces on the upper and lower wall, which causes some particles such as the PLTs to adhere even when not activated, and drag along. A fix would be to include contact forces on the walls to a certain degree to prevent this. I believe this project was very helpful in seeing and applying multiple forces into multiple particles, while no proper data structure techniques were implemented to save computation time, the concept in class was a great introduction for future projects in relation to particles. However, the most challenging part was the incredible amount of variables, functions and forces to take into account, bringing to attention the importance of segmentizing the work and creating a proper checklist of what needs to be applied for the project.